Adjuster CAN Protocol Specification

Version: V1.3

CAN NodeID：uint32_t，Little-endian（LSB first）

1. Design Update Summary

This version simplifies the motion control protocol based on system-level decisions:
1) Motion command from Device to Adjuster only carries 32-bit step count.
2) Speed and motor current are treated as global parameters configured separately.
3) Adjuster reports status and remaining steps only.

2. CAN ID Definition (Unchanged)

0x120  M→S  Motion Control Command (Step Only)
0x121  S→M  Status Feedback
0x122  M→S  Global Parameter Set
0x123  S→M  Parameter / EEPROM Report

3. Motion Control Command (0x120)

Purpose: Command adjuster to move by a given number of steps.
Speed, acceleration and motor current are pre-configured global parameters.

Data Format (8 Bytes, Classic CAN):

Byte0-3 : TargetNodeID (uint32)
Byte4   : AxisMask (bit0=X, bit1=Y)
Byte5   : Flags (bit0=Direction, bit1=Immediate)
Byte6-7 : Reserved

Extended Payload:
Byte8-11 : StepCount (int32, signed)

Positive value = forward
Negative value = reverse

4. Status Feedback (0x121)

Purpose: Report current motion state and remaining steps.

Data Format (16 Bytes):

Byte0-3 : SourceNodeID (uint32)
Byte4   : AxisMask
Byte5   : Status
Byte6   : ErrorCode
Byte7   : Reserved
Byte8-11 : RemainingSteps (int32)
Byte12-15: CurrentPosition (int32)

5. Status Bit Definition

bit0 = Moving
bit1 = Target Reached
bit2 = Error

6. Communication Example

Example: Device commands Adjuster NodeID 0x12345678 to move X axis by +2000 steps.

CAN ID : 0x120
Data (hex):
D9 A8 64 17   01   01   00 00   D0 07 00 00

Explanation:
StepCount = 2000

Example: Adjuster reports still moving, 800 steps remaining.

CAN ID : 0x121
Data (hex):
78 56 34 12   01   01   00   00   20 03 00 00   B0 04 00 00

RemainingSteps = 800
CurrentPosition = 1200