PC ↔ Device / Device ↔ Module Communication Protocol

This document defines the byte-level communication protocols for the PC → Device → Module system.
All multi-byte integers use Little Endian (LE) encoding.
USB HID frames are fixed at 64 bytes; unused bytes MUST be filled with 0x00.

1. PC-Side Device Filtering & Discovery

On the PC side, USB HID devices are first filtered by Vendor ID (VID) and Product ID (PID)
before any protocol-level handshake is performed.

USB Device Filter (PC Application):

Vendor ID (VID):  0x0483   // STMicroelectronics
Product ID (PID): 0xA7D1   // Custom product ID for this project

Only HID devices matching BOTH VID and PID shall be enumerated and opened.

Example (C#):

private const int FilterVendorId  = 0x0483;
private const int FilterProductId  = 0xA7D1;

After VID/PID filtering, the application shall perform an application-level discovery
by sending CMD_INFO / GET_DEVICE_INFO to validate that the opened HID device
implements the expected protocol.

2. Common Conventions

Frame Header: 0xA5
USB Frame Size: 64 bytes (fixed)
Endianness    : Little Endian (LE)
Reserved      : Fill with 0x00

Direction:
  DIR_PC_TO_MCU  = 0x00
  DIR_MCU_TO_PC  = 0x01

Primary Commands:
  CMD_INFO   = 0x01   // discovery, device info, module list
  CMD_MOTION = 0x10   // XY motion and remaining step report
  CMD_PARAM  = 0x11   // (reserved) parameters
  CMD_STATUS = 0x12   // (reserved) health/status

3. PC ↔ Device (USB HID)

3.1 USB: Get Device Info

Request (PC → Device): CMD_INFO / SUBCMD_GET_DEVICE_INFO (0x01)

Byte 0      0xA5
Byte 1      CMD_INFO (0x01)
Byte 2      DIR_PC_TO_MCU (0x00)
Byte 3      seq_L
Byte 4      seq_H
Byte 5      SubCmd = 0x01
Byte 6..63  Reserved = 0

Response (Device → PC): CMD_INFO / SUBCMD_GET_DEVICE_INFO (0x01)

Byte 0      0xA5
Byte 1      CMD_INFO (0x01)
Byte 2      DIR_MCU_TO_PC (0x01)
Byte 3      seq_L
Byte 4      seq_H
Byte 5      SubCmd = 0x01
Byte 6      StatusCode          // 0x00 = OK
Byte 7      Reserved

Byte 8..11  DeviceNodeID (uint32, LE)
Byte 12..19 DeviceName (ASCII, 8 bytes)

Byte 20..23 FirmwareVersion (uint32)
             [31:24]=major
             [23:16]=minor
             [15:0 ]=patch

Byte 24     MfgYear
Byte 25     MfgMonth
Byte 26     MfgDay
Byte 27     Reserved

Byte 28..63 Reserved = 0

3.2 USB: Get Module List (Paged)

Request (PC → Device): CMD_INFO / SUBCMD_GET_MODULE_LIST (0x02)

Byte 0      0xA5
Byte 1      CMD_INFO (0x01)
Byte 2      DIR_PC_TO_MCU (0x00)
Byte 3      seq_L
Byte 4      seq_H
Byte 5      SubCmd = 0x02
Byte 6      StartIndex
Byte 7      Reserved
Byte 8..63  Reserved = 0

Response (Device → PC): CMD_INFO / SUBCMD_GET_MODULE_LIST (0x02)

Byte 0      0xA5
Byte 1      CMD_INFO (0x01)
Byte 2      DIR_MCU_TO_PC (0x01)
Byte 3      seq_L
Byte 4      seq_H
Byte 5      SubCmd = 0x02
Byte 6      StatusCode              // 0x00 = OK
Byte 7      TotalModuleCount

Byte 8      StartIndexEcho
Byte 9      ReturnedCount           // 0..2
Byte 10..11 Reserved

Byte 12..31 ModuleEntry[0] (20 bytes)
Byte 32..51 ModuleEntry[1] (20 bytes)
Byte 52..63 Reserved = 0

3.3 USB: XY Motion Command

Request (PC → Device): CMD_MOTION (0x10)

Byte 0      0xA5
Byte 1      CMD_MOTION (0x10)
Byte 2      DIR_PC_TO_MCU (0x00)
Byte 3      seq_L
Byte 4      seq_H

Byte 5..8   TargetNodeID (uint32, LE)
Byte 9      AxisMask     // bit0=X, bit1=Y
Byte 10     MotionMode   // 0x00=Relative, 0x01=Absolute, 0x02=Home
Byte 11     Reserved

Byte 12..15 X_TargetStep (int32, LE)
Byte 16..19 Y_TargetStep (int32, LE)
Byte 20..63 Reserved = 0

3.4 USB: XY Remaining Step Report

Report (Device → PC): CMD_MOTION (0x10)

Byte 0      0xA5
Byte 1      CMD_MOTION (0x10)
Byte 2      DIR_MCU_TO_PC (0x01)
Byte 3      seq_L
Byte 4      seq_H

Byte 5..8   NodeID (uint32, LE)
Byte 9      Reserved
Byte 10     StatusFlags
             bit0 = running
             bit1 = done
             bit2 = error
Byte 11     Reserved

Byte 12..15 X_RemainingStep (uint32, LE)
Byte 16..19 Y_RemainingStep (uint32, LE)
Byte 20..63 Reserved = 0

4. Device ↔ Module (CAN FD, FDBRS)

CAN IDs:
0x120  Device → Module  Motion Command
0x121  Module → Device  Motion Status
0x122  Device → Module  Parameter Set (reserved)
0x123  Module → Device  Parameter Report (reserved)
0x203  Module → Device  Heartbeat / Identity (500ms)

CAN Motion Command (ID 0x120, 16 bytes):

Byte 0..3   TargetNodeID (uint32, LE)
Byte 4      AxisMask
Byte 5      MotionMode
Byte 6..7   Reserved
Byte 8..11  X_TargetStep (int32, LE)
Byte 12..15 Y_TargetStep (int32, LE)

CAN Motion Status (ID 0x121, 16 bytes):

Byte 0..3   NodeID (uint32, LE)
Byte 4      StatusFlags
Byte 5      ErrorCode
Byte 6..7   Reserved
Byte 8..11  X_RemainingStep (uint32, LE)
Byte 12..15 Y_RemainingStep (uint32, LE)

CAN Heartbeat / Identity (ID 0x203, 20 bytes):

Byte 0..3   NodeID (uint32, LE)
Byte 4..11  ModuleName (ASCII, 8 bytes)
Byte 12..15 FirmwareVersion (uint32)
Byte 16     MfgYear
Byte 17     MfgMonth
Byte 18     MfgDay
Byte 19     ModuleType

Appendix A: Complete Communication Sequence (Text Flow Diagram)

This section describes the complete end-to-end communication flow between PC, Device, and Modules.
The flow is written in a strict textual sequence form, suitable for protocol validation,
Copilot prompting, and future transport replacement (USB → TCP/UDP).

A.1 System Startup & Device Discovery

PC Application Start
 |
 |--[USB HID Enumerate]
 |     Filter by VID = 0x0483
 |     Filter by PID = 0xA7D1
 |
 |--[Open HID Device]
 |
 |--[CMD_INFO / GET_DEVICE_INFO]
 |     PC  -> Device
 |
 |--[Device Info Response]
 |     Device -> PC
 |     (NodeID, DeviceName, FW, Date)
 |
 |--[Validate Protocol Header]
 |     Header == 0xA5
 |
 V
Device Accepted

A.2 Module Enumeration (Module List)

PC Requests Module List
 |
 |--[CMD_INFO / GET_MODULE_LIST]
 |     StartIndex = 0
 |
 |--[Module List Response]
 |     TotalModuleCount = N
 |     ReturnedCount = 0..2
 |
 |--[Repeat]
 |     StartIndex += ReturnedCount
 |     Until StartIndex >= TotalModuleCount
 |
 V
PC Has Full Module List

A.3 User Operation & Motion Control

User Selects Module (by NodeID)
 |
 |--[CMD_MOTION]
 |     TargetNodeID
 |     AxisMask (X/Y)
 |     MotionMode
 |     TargetSteps
 |
 |-- PC -> Device
 |
 |-- Device Forwards to CAN (ID 0x120)
 |
 |-- Module Executes Motion
 |
 V
Motion Running

A.4 Motion Feedback & Completion

While Motion Running:
 |
 |--[CAN Motion Status 0x121]
 |     Module -> Device
 |
 |--[USB Remaining Step Report]
 |     Device -> PC
 |
 |--[Update UI]
 |     Disable buttons if running
 |
Repeat until:
 |
 |-- StatusFlags.done == 1
 |
 V
Motion Completed

A.5 Background Heartbeat & Presence Monitoring

Periodic (500ms):
 |
 |--[CAN Heartbeat 0x203]
 |     Module -> Device
 |
 |--[Update Module Presence]
 |
If heartbeat timeout:
 |
 |-- Mark module offline
 |-- Block motion commands
 |
 V
System Health Maintained

Appendix B: Module Extended Functions (CAN FD)

This appendix defines extended module-side functions implemented on the Adjuster module.
All commands are sent from Device → Module via CAN FD (FDBRS).
Unless otherwise stated, parameters are persistent (stored in Flash).

B.1 Additional CAN ID Allocation

0x124  Device → Module  Motor Electrical & Motion Parameters
0x125  Device → Module  Module Management / Factory Functions
0x126  Device → Module  Identify / Auto-Run Control

B.2 Set Motor Current (X/Y Independent)

CAN ID: 0x124
Payload (16 bytes):

Byte 0..3   TargetNodeID (uint32, LE)
Byte 4..5   X_Current_mA (uint16, LE)
Byte 6..7   Y_Current_mA (uint16, LE)
Byte 8      SaveToFlash   // 0=RAM only, 1=Save to Flash
Byte 9..15  Reserved

B.3 Set Step Frequency (X/Y Shared)

CAN ID: 0x124
Payload (alternative use):

Byte 0..3   TargetNodeID (uint32, LE)
Byte 4..5   StepFrequency_Hz (uint16, LE)  // Range: 100–500
Byte 6..7   Reserved
Byte 8      SaveToFlash
Byte 9..15  Reserved

B.4 Set Acceleration / Deceleration

CAN ID: 0x124
Payload (alternative use):

Byte 0..3   TargetNodeID (uint32, LE)
Byte 4..5   Accel_StepPerS2 (uint16, LE)
Byte 6..7   Decel_StepPerS2 (uint16, LE)
Byte 8      SaveToFlash
Byte 9..15  Reserved

B.5 Rename Module (8-byte ASCII)

CAN ID: 0x125
Payload (16 bytes):

Byte 0..3   TargetNodeID (uint32, LE)
Byte 4..11  ModuleName (ASCII, 8 bytes)
Byte 12     SaveToFlash
Byte 13..15 Reserved

B.6 Set Factory Position (Zero Calibration)

CAN ID: 0x125
Payload (16 bytes):

Byte 0..3   TargetNodeID (uint32, LE)
Byte 4      Action = 0x01   // Set current position as (0,0)
Byte 5      SaveToFlash = 1
Byte 6..15  Reserved

B.7 Identify Module

CAN ID: 0x126
Payload (16 bytes):

Byte 0..3   TargetNodeID (uint32, LE)
Byte 4      Action = 0x01   // Identify
Byte 5..15  Reserved

Identify behavior:
  X-axis move -100 steps
  Delay 1 second
  X-axis move +100 steps

B.8 Go To Factory Position

CAN ID: 0x125
Payload (16 bytes):

Byte 0..3   TargetNodeID (uint32, LE)
Byte 4      Action = 0x02   // Go to factory position
Byte 5..15  Reserved

B.9 Relative Position Reporting

Extension to CAN Heartbeat (ID 0x203):

Byte 20..23 X_RelativePos (int32, LE)
Byte 24..27 Y_RelativePos (int32, LE)

If heartbeat length < 28 bytes, Device shall assume position unknown.

B.10 Set Factory Date

CAN ID: 0x125
Payload (16 bytes):

Byte 0..3   TargetNodeID (uint32, LE)
Byte 4      Year  (uint8)   // e.g. 25 for 2025
Byte 5      Month (uint8)
Byte 6      Day   (uint8)
Byte 7      SaveToFlash = 1
Byte 8..15  Reserved

B.11 Auto Run (Factory Aging)

CAN ID: 0x126
Payload (16 bytes):

Byte 0..3   TargetNodeID (uint32, LE)
Byte 4      Enable
             0x00 = Stop
             0x01 = Start
Byte 5..15  Reserved

Auto-run behavior is module-defined and repeats
until Stop command is received.

B.12 Motion Status Extension (0x121)

CAN Motion Status (ID 0x121) Extension:

Byte 5  RejectReason (NEW)
  0x00 = None
  0x01 = Limit hit (motion rejected)
  0x02 = Invalid state
  0x03 = Parameter error

When RejectReason != 0x00:
  - Motion shall not start
  - Device shall notify PC

Appendix C: Parameter Query & Extended Status (Optional, Recommended)

This appendix defines OPTIONAL but RECOMMENDED extensions for long-term maintainability.
These additions do NOT break existing implementations and are backward-compatible.

C.1 Get Module Parameters (Query)

CAN ID: 0x127   // Device → Module
Purpose: Query current effective parameters from module

Payload (8 bytes):
Byte 0..3   TargetNodeID (uint32, LE)
Byte 4      QueryMask
             bit0 = Motor current
             bit1 = Step frequency
             bit2 = Accel/Decel
             bit3 = Factory position
             bit4 = Factory date
Byte 5..7   Reserved

C.2 Module Parameter Report

CAN ID: 0x128   // Module → Device
Purpose: Report current parameters

Payload (32 bytes):
Byte 0..3   NodeID (uint32, LE)

Byte 4..5   X_Current_mA (uint16, LE)
Byte 6..7   Y_Current_mA (uint16, LE)

Byte 8..9   StepFrequency_Hz (uint16, LE)

Byte 10..11 Accel_StepPerS2 (uint16, LE)
Byte 12..13 Decel_StepPerS2 (uint16, LE)

Byte 14..17 FactoryPos_X (int32, LE)
Byte 18..21 FactoryPos_Y (int32, LE)

Byte 22     MfgYear
Byte 23     MfgMonth
Byte 24     MfgDay

Byte 25..31 Reserved

C.3 Extended Motion / AutoRun Status Flags

Extension to CAN Motion Status (ID 0x121):

Byte 4  StatusFlags (extended)
  bit0 = running
  bit1 = done
  bit2 = limit_hit
  bit3 = error
  bit4 = auto_running   // NEW
  bit5 = busy           // NEW (reject normal motion)
  bit6..7 reserved

Byte 5  RejectReason
  0x00 = None
  0x01 = Limit hit
  0x02 = Busy (AutoRun active)
  0x03 = Invalid state
  0x04 = Parameter error

Behavior rules:
- If auto_running = 1, module SHALL reject CMD_MOTION commands.
- Rejected motion MUST return RejectReason != 0x00.
- Device SHALL forward rejection to PC.