# Stage Heartbeat Protocol Specification

Version: V1.2

Module: Adjuster / Stage

Bus: CAN-FD (FDBRS)

Payload Length: 32 Bytes

# 1. Overview

This document defines the CAN-FD heartbeat protocol used by the Stage/Adjuster module. The heartbeat frame is a fixed-length status broadcast packet, used for device identification, runtime status monitoring, and motor operating status reporting.

# 2. Frame Definition

CAN ID: 0x203 (Standard ID)

Direction: Module → Device (Broadcast)

Period: 1000 ms (Idle)

Data Length: 32 Bytes

# 3. Payload Layout (32 Bytes)

| Byte Index | Field | Size (Bytes) | Description |
| --- | --- | --- | --- |
| 0 | module_type | 1 | Module type identifier (MODULE_ADJUSTER = 0x12) |
| 1–4 | node_id | 4 | Unique Node ID (LSB first) |
| 5–12 | name[8] | 8 | ASCII module name, null padded |
| 13 | fw_hw | 1 | Hardware version |
| 14 | fw_dd | 1 | Firmware build / revision |
| 15 | fw_mm | 1 | Firmware minor version |
| 16 | fw_yy | 1 | Firmware major version |
| 17 | mfg_year | 1 | Manufacture year (25 = 2025) |
| 18 | mfg_month | 1 | Manufacture month (1–12) |
| 19 | mfg_day | 1 | Manufacture day (1–31) |
| 20 | status_flags | 1 | Runtime status flags |
| 21 | error_flags | 1 | Error flags (sticky) |
| 22 | motor_hold_current | 1 | Motor hold current ratio (0–100 %, relative to set current) |
| 23–24 | x_pos | 2 | X-axis position (step) |
| 25–26 | y_pos | 2 | Y-axis position (step) |
| 27 | x_cur | 1 | X-axis running current ratio (0–100 %) |
| 28 | y_cur | 1 | Y-axis running current ratio (0–100 %) |
| 29 | heartbeat_cnt | 1 | Heartbeat counter |
| 30 | motor_speed | 1 | Motor speed (unit: 10 step/s) |
| 31 | crc8 | 1 | CRC8 checksum (reserved) |

# 4. status_flags Bit Definition

| Bit | Name | Description |
| --- | --- | --- |
| 0 | RUNNING | Motor is running |
| 1 | MOTION_DONE | Last motion completed |
| 2 | HOMED | Homing completed |
| 3 | BUSY | Module busy |
| 4 | ENABLED | Motor driver enabled |
| 5 | ESTOP | Emergency stop active |
| 6 | WARNING | Non-fatal warning |
| 7 | RESERVED | Reserved, must be 0 |

# 5. error_flags Bit Definition

| Bit | Name | Description |
| --- | --- | --- |
| 0 | X_LIMIT_ERR | X-axis limit error |
| 1 | Y_LIMIT_ERR | Y-axis limit error |
| 2 | X_OVERCURRENT | X-axis overcurrent |
| 3 | Y_OVERCURRENT | Y-axis overcurrent |
| 4 | DRIVER_FAULT | Motor driver fault |
| 5 | COMM_ERROR | Communication error |
| 6 | CONFIG_ERROR | Configuration error |
| 7 | FATAL_ERROR | Fatal error, reset required |